| Tender Number / Description: |  |
| --- | --- |
| - Service Scope Deliverables: | Bidder Compliance Comments to deliverables/requirements (Including Attachment Reference) |
| Detail specific services and deliverables, including scope, objectives, and timelines. Include any unique methodologies or approaches. BIDDER shall provide services including material and personnel to cover the scope mentioned below: - Engineering and preparations of lifting plans - Crane, forklift, Cherry Picker services - Bidder should provide suitable Crane for lifting operations as per provided Lifting plans - Bidder should provide suitable Forklift for lifting operations as per provided Lifting plans - All machinery has to have valid inspection certificates, MES licenses, - Modular scaffolding services - Bidder should provide easy-access scaffolding included but not limited with stairs, decking, platforms, bridging, guardrails. - Bidder should be able to prepare design, 3D modeling(if requested) and calculating static and dynamic load. - Usage only stainless steel/aluminum materials. - All components should comply with EN or BS standards. - Bidder must have available stock of minimum 100Te. - Planning & Control of lifting operations - Qualified Senior Lifting Supervisor - Rigging Team consisting of 1x Rigger level 3, 1x Banksman, 1x Slinger - Rigging Loft equipment - Bidder to provide 2x Rigging loft containers full of required up-to-date certified rigging and lifting equipment to avoid any delays on daily processes, including: Shackles, spreader beams, slings, hoists, master links etc. - Warehouse services - Bidder has to have minimum of 5 20ft containers for storing subcontractor’s equipments, materials, tools and all relevant items. - 2x Storeman for proper goods in/out controlling/ - Manpower Support - Rope Access Team - Mechanical/Fitter Team - Labor Team - HSE Team (If Required) - Training Center - NDT Inspection - Inspection ground level and rope access: - Inspect equipment according to API RP 8B (Recommended Practice for Procedures for Inspections, Maintenance, Repair, and Remanufacture of Hoisting Equipment). - Inspect handling equipment in satisfaction with the full intent of the guidelines specified in the latest edition of API Recommended Practice 7L. - Inspection, Destruction testing and recertification of wire ropes: - Conduct inspection, destruction testing and recertification of wire ropes as per API RP 2D (operation and maintenance of offshore cranes) and API RP 9A (Specification of wire rope) or alternative - Lifting equipment and accessories inspection ground level and rope access: - Adhere to all requirements of local legislation and applicable/recognized industry standards (i.e. LOLER) - Carry out a thorough examination of lifting equipment - Be responsible for color-coding and certification of examined equipment - Carry out periodic thorough examinations as per an approved written scheme of examination and tests in accordance with approved criteria - Immediately notify the Client and remove from service any lifting appliance/accessories which are: - Sub-standard - Modified - Home-made - Defective / not fit for purpose - Tank Inspections under DNV/ABS class supervision: - Have relevant certification to conduct inspection in accordance with class requirement (DNV, ABS etc.) - Non-Destructive Testing (NDT) and Ultrasonic thickness (UT) gauging shall be capable of accurately inspecting surface even when they are coated with paint or other coatings. - All tanks (where possible) and voids shall be entered and visually inspected. Where visual inspections reveal corrosion and/or deterioration to any primary hull plating and scantlings, such areas shall undergo UT thickness gauging - Welding Joints, as per Class inspection scope, shall be NDT checked against any cracks and defects. - All machinery spaces shall be visually inspected - All jack up cantilever packages, including the drill floor, sub-structure, sub-base, etc., shall be visually inspected. - All accessible hull exterior surfaces shall be visually inspected. This shall include all accessible columns and bracing, as well as the underside of the main deck structure on semi-submersible rigs. On jack up rigs, the exterior hull inspection shall include all accessible areas of all leg wells, side shell, and hull bottom plating. - All accessible areas of chain lockers and hawse pipes shall be visually inspected on all moored rigs. - All crane booms shall be visually inspected for coating system breakdown and UT gauged where corrosion or deterioration has been incurred. - All accessible portions of carbon steel and alloy steel high pressure piping systems (including but not limited to high pressure mud, cement, well testing, and choke and kill system piping) shall be UT gauged. All gauging shall be recorded in the inspection report and compared with applicable classification society and flag state rules, as well as any local laws that may govern the minimum allowable wall thickness values until such time as a company standard is issued. - Accessible areas inside the rig floor structure that specifically support the drawworks, rotary table, derrick and set-back areas shall be visually inspected and UT gauged where corrosion or deterioration has been incurred. - All accessible areas above the main deck shall be visually inspected including but not limited to crane pedestals, helideck and support structure, lifeboat davits and support structures, jacking gear units and brace beams, mooring winch/windlass support structure, quarter’s structure, shaker houses, and all watertight closures. - Tank vents shall be randomly inspected for proper functionality and to confirm that they are not blocked with foreign material. Fuel tank vents shall be inspected to confirm that flame screens are fitted and are of the proper mesh size (30 x 30 minimum mesh size). - All accessible decks shall be visually inspected and UT gauged where corrosion or deterioration has been incurred. - All accessible surfaces of piping within tanks or voids shall be visually inspected and UT gauged where corrosion or deterioration has been incurred. - Provide report into each specific tank, area, or system inspected, with detailed inspection information, up to Class acceptance, including but not limited to photographs, UT gauge readings, cracks, defects details and other information that supports the conclusions and recommendations in the inspection report. | Engineering and preparation of lifting plans & Route Study of oversized Cargo - Engineer to conduct site rigging study to determine lift plan requirements - Develop detailed engineered lift plans in Auto-Cad format, method statements and risk assessments to ensure company safe operation in accordance with company standards and procedures. - Identification of critical, complex and unique lifts, assessment of crane and rigging requirements including Mechanical Handling. - Rigging equipment requirements & configurations to be integrated into the lift plans. - Calculates wind sail to ensure safe maximum wind speed in ms_2 (Recommended nominal wind speed 11ms_2) - Authorization of all lifting plans & risk assessments for all categories of lifts. - Complete oversite of all Rigging & Lifting personnel and all Lifting and Mechanical Handling activities. - Develop Route study analysis for oversized transport operations. Crane, Forklift, And MEWP Services - Crane-age requirements to be integrated into engineered lift plans. - Crane to be 3rd party inspected and certified prior to mobilization to site supported with a competent, certified crane operator. - Forklift to be 3rd Party inspected and certified prior to mobilization to site supported by a competent, certified forklift operator. - Forklift make & model specifics to be determined by the client. - Mobile Elevated Work Platforms (MEWP) to be 3rd party inspected and certified prior to mobilization to site supported with a competent, certified MEWP operator. - All work at height over 1.6 meters shall be conducted in accordance with company HSE requirements. (100% tie-off) - MEWP make and model specifics to be determined by the client. Planning & Control of Lifting Operations - Lifting Supervisor Level 3 - All site lifting operations to be overseen by company supplied certified level 3 lifting supervisor. - Responsibilities include but not limited to (below). - Ensuring crane configurations are in accordance with supplied engineered lift plan. - Ensuring rigging configurations are in accordance with supplied engineered lift plans. - Ensure rigging & lifting equipment has been inspected and deemed safe for use prior to all lifting operations. - Conduct site Toolbox Talk with all Rigging, lifting and mechanical personnel prior to commencing operations. - Ensure Tag Lines are utilized for all lifting operations. - Mechanical Handling operations are carried out in accordance with company supplied Mechanical Handling plans. - Ensure all Method statements are followed in accordance with company supplied method statements. - Ensure all lifting and rigging operations are conducted in compliance with both company and client rigging & lifting procedures. - Ensure all rigging & lifting personnel operate in a safe & efficient manner, which is in line with company procedures. - Ensure all lifting and rigging operations are conducted with a proactive attitude towards health, safety and the environment. - Always maintain good housekeeping. - Conduct all operations in accordance with company & client HSE site procedures. - Report any Near Misses or LTA, s to your line Manager & Site supervisor immediately. - Ensure proper PPE is always worn on site. - Always represent the company in a professional manner. - Rigger Level 2 - Ensure rigging & lifting equipment has been inspected and deemed safe for use prior to all lifting operations. - Ensure Tag Lines are utilized for all lifting operations. - Ensure Mechanical Handling operations are carried out in accordance with company supplied Mechanical Handling plans. - Ensure all Method statements are followed in accordance with company supplied method statements. - Ensure all lifting and rigging operations are conducted with a proactive attitude towards health ,safety and the environment. - Always maintain good housekeeping. - Report any Near Misses or LTA, s to your line Manager & Site supervisor immediately. - Ensure all rigging & lifting personnel operate in a safe & efficient manner, which is in line with company procedures. - Conduct all operations in accordance with company & client HSE site procedures. - Ensure proper PPE is always worn on site. - Always represent the company in a professional manner. - Banksman Level 2 - Ensure area is barriered off with sufficient signage prior to carrying out any lifting operations. - Ensure Tag Lines are utilized for all lifting operations. - Ensure rigging & lifting equipment has been inspected and deemed safe for use prior to all lifting operations. - Banksman to wear Hi-Vis vest during all lifting operations - Ensure proper PPE is always worn on site. - Ensure all personnel are out of “The Line of Fire” prior to lifting. - Maintain good communications at all times with the crane operator. - Only use Internationally approved and recognized hand signals. - Ensure area is clear of unauthorized personnel prior to commencing lifting operations. - Always ensure good visibility with the crane operator. - Never carry out any blind lifts unless they are planned in advance and cleared by your line supervisor. - Never conduct any blind lifts without radio communications between Banksman & Crane operator (separate radio channel is mandatory). - Ensure all lifting and rigging operations are conducted in compliance with both company and client rigging & lifting procedures. - Never let personnel walk under a suspended load. - Never conduct lifting whilst other trades are working within the lift radius (Symops) unless previously agreed by the client and lifting supervisor. - Conduct all operations in accordance with company & client HSE site procedures. - Ensure all lifts are conducted in a safe & efficient manner, which is in line with company procedures. - Always maintain good housekeeping. All barriers & signage to be removed after lifting operations are completed. - Always represent the company in a professional manner. |
| - Personnel Requirements: |  |
| Define required qualifications, such as specific degrees or certifications. Include experience levels and any specialized skills needed - All personnel involved in activities mentioned in section 1 of this tender document must be trained, qualified and certified as per recognized international standard(s) and/or relevant Azerbaijan regulations. Scaffolding - NDE inspections shall be carried out by a person who is qualified to American Standards for Non-Destructive Testing (ASNT) Level II requirements or equal. - Inspection of Lifting Equipment and Accessories shall be carried out by LEEA certified Inspector to carry out - For rope access activities, BIDDER shall ensure availability of IRATA Level III, II and I rope access specialists with valid certification. - BIDDER has full responsibility for the selection and directing of competent, trained and certified personnel engaged in the activities mentioned in section 1 of this document. Furthermore, the BIDDER has full responsibility to ensure that personnel assigned to abovementioned activities are adequately trained to recognize and mitigate potential hazards inherent in tasks. - BIDDER shall ensure that their personnel wear the following minimum certified and always complying with ANSI or equivalent standard PPE whilst working on site: safety helmet, safety footwear, safety glasses, high visibility coverall, gloves and certified fall arrest harness. All PPE must be used in accordance with the site specific rules, job risk assessment and valid work permit. |  |
| - Service Specifications: |  |
| - BIDDER should provide a company brochure or document showing the company capabilities and achievements, successful projects with references from previous clients, particularly those in the oil and gas projects. - BIDDER should demonstrate the method of monitoring HSE cases and provide HSE records, particularly HSE incident statistics and if requested copies of incident reports in previous oil and gas services. - BIDDER shall carry out mandatory periodic inspections by trained and certified inspectors according to recognized international, BS or EN standards and local Azerbaijan regulations to ensure that they meet all relevant HSE and QA/QC requirements. Please provide company procedure covering periodic inspections. - BIDDER to provide a copy of operational procedures and method statement for the scope of work |  |
| - Equipment and Materials: |  |
| Specify types and quality standards of equipment and materials required. - All equipment provided by BIDDER shall have valid certification and inspection records according to OEM requirements and recommendations. BIDDER should be able to provide evidence about certification, inspection and maintenance of utilized equipment - BIDDER shall be able to demonstrate that it is capable to provide sufficient quantity of certified equipment according to service scope. - BIDDER shall provide certified/calibrated inspection equipment which suitable for the scope |  |
| - Training and Competence: |  |
| Detail mandatory training programs and competence levels for personnel. Include periodic assessment and certification processes. - BIDDER shall provide a training matrix showing the training scheme, training categories and current status of recorded training for all personnel. - BIDDER shall ensure that all personnel assigned to tasks are competent, trained and certified according to international standards or local Azerbaijan regulations for abovementioned activities. BIDDER is required to provide verifiable evidence about training and competence for all personnel. - BIDDER shall ensure and has full responsibility for selecting and directing competent, trained and certified personnel engaged in activities specified in tender documents. |  |
| 6. Safety and Certification: |  |
| Specify safety protocols and standards. List industry-specific certifications and compliance requirements List all necessary certifications and accreditations, both for personnel and the project. - The bidder should highlight the number of competent personnel employed in their Company to fulfil each service scope item (Add to the right-hand column for each service scope) - The bidder should describe their Company and personnel certification to fulfil each service scope (Add to the right-hand column for each service scope). - BIDDER shall ensure that all assigned personnel have required full PPE for work, in line with ANSI standards or equivalent - BIDDER shall ensure that all assigned personnel are competent, trained and certified according to the relevant recognized international standards - BIDDER shall have an emergency response plan for the job/work onsite and be fully responsible for their implementation. - BIDDER shall be aware that the possession or consumption of cigarettes, electronic cigarettes (E-Cigarettes), alcohol and/or illegal drugs is strictly prohibited on all Client facilities and will result in disciplinary action which may result in dismissal of contract. Please confirm acceptance of this policy and describe how you are going to control compliance with this policy for your personnel? - BIDDER should provide: - Copy or description of Quality Management System that comply to ISO 9001 standard. If available, please provide a copy of ISO 9001 certificate or equivalent. - Copy or description of Health and Safety Management System that complies to ISO 45001 standard. If available provide copy of ISO 45001 certificate or equivalent. - Copy or description of Environmental Management System that complies with ISO 14001 standard. If available, please provide a copy of ISO 14001 certificate or equivalent. - Copy of ISO 17020 certificate or equivalent. - Copy of license from MES or MOE - Copy of IRATA membership or equivalent - Copy of LEEA membership or equivalent - Copy of ABS/DNV – Certificate of Recognized Service Supplier certificate. - A copy of the signed quality policy. - A copy of the quality objectives. - A copy of signed H&S policy. - A copy of signed environmental policy. - A copy of relevant certifications, accreditations, or memberships. - Evidence is that BIDDER has a minimum of 5 years’ experience in delivering of all services mentioned in scope. - Availability of Motion Kinetic Certification Software or alternative. - Records of professional training and qualifications showing the years of experience of supervisors and all personnel involved in related activities. - OEUK Medical requiired for all personnel. - A copy of the company training matrix showing the recorded training scheme, training categories and current status of training for all personnel. - Evidence that all equipment/materials have valid manufacturer certificates. - Local HSE accreditation/operating certificate(s). |  |
| 7. Operational Integration and Compliance: |  |
| Describe integration with operations and compliance with regulatory standards. Include data management and reporting procedures. BIDDER shall ensure: - Full integration of team activities into the HSE management plan. - Adherence to work permit procedures and Job Risk Assessment (JRA). - Daily log of tasks onsite, maintained by the assigned supervisors. - Attendance of required personnel on daily meetings with project team and distribute daily progress report to relevant Client personnel. |  |
| 8. Experience and Working Conditions: |  |
| Specify minimum experience requirements for key project roles. Detail working conditions, including any environmental or logistical considerations. - BIDDER and all personnel involved in mentioned activities must have at least 5 years experience supported by evidences. All personnel shall have valid OEUK medical survival certification provided by an OPTIO certified organization and maintained in renewal cycles as applicable to OPTIO guidelines and reference letters. - BIDDER should be able to ensure detail monitoring of the working conditions, including any environmental or logistical considerations. Also, will be able to cover defined working hours. Please provide copies of procedures covering this |  |
| 9. Subcontracting Policies: |  |
| If subcontracting is allowed, define the conditions under which it can occur and how subcontractors will be managed. - Subcontracting is allowed up to 25% of total value |  |
| 10. Sustainability and Environmental Impact: |  |
| Include requirements or guidelines for environmental sustainability, especially if the project has significant environmental impacts - BIDDER should have implemented environmental management systems and evidence/procedure environmental management is always considered before, during and after all activities included in scope of this tender. |  |
| 11. Local Content and Community Engagement: |  |
| If applicable, include requirements or guidelines for the inclusion of local content and engagement with local communities - Not applicable | N/A |